# Arduino LED & Potentiometer – Tinkercad Circuit Documentation

## 1. Aim / Objective

To design and simulate an Arduino UNO circuit in Tinkercad that controls multiple LEDs using digital output pins and reads a potentiometer value using an analog input.

## 2. Components Required

- Arduino UNO

- Breadboard

- 6 × LEDs (any colors)

- 6 × Resistors (220Ω or 330Ω)

- 1 × Potentiometer (10kΩ)

- Jumper wires

- USB cable (for Arduino power

## 3. Circuit Description

The Arduino UNO supplies 5V power to the breadboard. Each LED is connected to a digital pin through a current-limiting resistor. The potentiometer provides a variable voltage to analog pin A0, which the Arduino reads to control the LED output pattern.

## 4. Power Connections

| Arduino Pin | Breadboard Connection |
| --- | --- |
| 5V | Positive rail (+) |
| GND | Negative rail (–) |

## 5. LED Connections

Each LED is connected as follows:

- Anode (+) → Arduino digital pin via 220Ω resistor

- Cathode (–) → GND rail

### Digital Pin Assignment

| Arduino Pin | LED |
| --- | --- |
| D2 | LED 1 |
| D3 | LED 2 |
| D4 | LED 3 |
| D5 | LED 4 |
| D6 | LED 5 |
| D7 | LED 6 |

## 6. Potentiometer Connections

| Potentiometer Pin | Connection |
| --- | --- |
| Left pin | 5V |
| Middle pin | A0 (Analog Input) |
| Right pin | GND |

The potentiometer acts as a voltage divider and sends a variable analog signal (0–5V) to the Arduino.

## 7. Working Principle

- Arduino powers the breadboard.

- The potentiometer position changes the voltage at pin A0.

- Arduino reads the analog value (0–1023).

- Based on this value, LEDs are turned ON or OFF.

- More LEDs glow as the potentiometer value increases.

## 8.Block Diagram

+-------------------+

| Power Supply     |

| (USB / 5V)         |

+---------+---------+

|

v

+-------------------+

| Arduino UNO     |

|                             |

| - Digital Pins       |-----> LED + Resistors (D2–D7)

| - Analog Pin A0  |<----- Potentiometer (10kΩ)

+---------+---------+

|

v

+-------------------+

| GND Reference |

+-------------------+

## 9. Arduino Program (Tinkercad Compatible)

int potPin = A0;

int leds[] = {2,3,4,5,6,7};

void setup() {

for(int i = 0; i < 6; i++) {

pinMode(leds[i], OUTPUT);

}

}

void loop() {

int potValue = analogRead(potPin);

int level = map(potValue, 0, 1023, 0, 6);

for(int i = 0; i < 6; i++) {

if(i < level)

digitalWrite(leds[i], HIGH);

else

digitalWrite(leds[i], LOW);

}

}

## 10. Applications

- LED level indicator

- Arduino basics learning

- Analog to digital conversion demo

- Tinkercad simulation practice

## 11. Conclusion

This Tinkercad circuit demonstrates how Arduino uses analog input to control multiple digital outputs. It is a beginner-friendly project for understanding LEDs, resistors, potentiometers, and Arduino programming.

# link

https://www.tinkercad.com/things/8n UagxXPiD9N-funky-jaagub?sharecode=XALLNy6A4fgzZ_g68h4_8rzKmT-Cv9XowqoES_A83kc