# INTERNAL COMPONENT& WORKING FLOW

INTRODUCTION

A plasma lighter (also called an arc lighter) creates an electric plasma arc instead of a flame. Below is a clear breakdown of its internal components and how it works step by step.

## 1. Main Internal Components

### 1. Rechargeable Battery

- Usually a 3.7V lithium-ion or lithium-polymer battery

- Stores energy for arc generation

- Charged via USB (Type-A, Micro-USB, or USB-C)

### 2. Charging Circuit

- Regulates safe charging of the lithium battery

- Protects against:

  - Over-charging

  - Over-discharging

  - Short circuits

- Often includes charge indicator LEDs

### 3. Power Control Button / Touch Sensor

- User interface to activate the lighter

- Sends a signal to the control circuit

- Some models use capacitive touch instead of a mechanical button

### 4. Control & Oscillator Circuit (PCB)

- The “brain” of the lighter

- Functions:

  - Converts low-voltage DC to high-frequency AC

  - Controls arc duration (auto-off safety, usually 5–10 seconds)

- Typically uses:

  - Transistors or MOSFETs

### 5. Step-Up Transformer (High-Voltage Transformer)

- Raises voltage from ~3.7V to several thousand volts

- Operates at high frequency (kHz range)

- Essential for creating plasma between electrodes

### 6. Electrodes (Arc Tips)

- Usually two or four metal electrodes

- Made from heat-resistant conductive alloys

- Spaced closely to allow arc formation

- Plasma arc temperature can exceed 1000–2000°C

### 7. Protective Casing & Insulation

- Heat-resistant plastic or metal shell

- Insulates user from high voltage

- Prevents accidental contact with electrodes

## 2. Working Flow (Step-by-Step Operation)

### Step 1: Activation

- User presses button or touches sensor

- Control circuit wakes up

### Step 2: Power Conversion

- Battery supplies low-voltage DC

- Oscillator circuit converts DC → high-frequency AC

### Step 3: Voltage Boosting

- AC current flows into step-up transformer

- Voltage increases to thousands of volts

- Current remains very low (safe but powerful)

### Step 4: Plasma Arc Formation

- High voltage jumps across electrode gap

- Air molecules ionize → plasma arc

- Visible purple/blue electric arc appears

### Step 5: Ignition

- Arc provides extreme localized heat

- Instantly ignites:

  - Paper

  - Cigarettes

  - Candles

  - Thin wood shavings

### Step 6: Auto Shut-Off

- After preset time (or button release):

  - Circuit cuts power

  - Arc extinguishes

- Prevents overheating and battery drain

## 3. Simplified Flow Diagram (Text)

Battery (3.7V DC)

↓

Control / Oscillator Circuit

↓

High-Frequency AC

↓

Step-Up Transformer

↓

High Voltage (kV range)

↓

Electrodes

↓

Plasma Arc (Heat)

## Plasma Lighter – Work Flow Diagram

### Simple Text-Based Work Flow Diagram

[User Presses Button / Touch Sensor]

↓

[Control Circuit ON]

↓

[Battery (3.7V DC Power)]

↓

[Oscillator / Switching Circuit]

(DC → High-Frequency AC)

↓

[Step-Up Transformer]

(Low Voltage → High Voltage)

↓

[High-Voltage Output]

↓

[Electrodes (Small Gap)]

↓

[Air Ionization]

↓

[Plasma Arc]

↓

[Ignition]